Sample Specification for Restraint of C909 PVC 4in – 12in

Restraint for AWWA C909 PVC Pipe shall consist of the following: The restraint shall be manufactured of ductile iron conforming to ASTM A536. The restraint devices shall be coated with MEGA-BOND. (For complete specifications on MEGA-BOND visit www.ebaa.com.) The combination of the restraint(s) and fasteners shall have a pressure rating to the full pressure rating of the pipe. The restraint shall have a two to one safety factor.

Restraint at Ductile Iron Fittings:

A split serrated ring shall be used to grip the plain-end of the pipe, a sufficient number of bolts shall connect the restraint ring to the ductile iron fitting. The restraint for mechanical joint fittings shall be the Series 19MJ00. The restraint for push-on fittings with restraining ears shall be the 19PF00.

Restraint Harness:

A split serrated ring shall be used to grip the plain-end of the pipe. A split serrated ring shall also be used to grip the barrel of the pipe behind the bell, and a sufficient number of bolts shall be used to connect the restraint rings. The combination shall be the Series 1900.

All Series manufactured by EBAA Iron, Inc., or approved equal.

Sample Specification for Restraint of C909 PVC 16in

Restraint for AWWA C909 PVC Pipe shall consist of the following: The restraint shall be manufactured of ductile iron conforming to ASTM A536. The restraint devices shall be coated with MEGA-BOND. (For complete specifications on MEGA-BOND visit www.ebaa.com.) The combination of the restraint(s) and fasteners shall have a pressure rating to the full pressure rating of the pipe. The restraint shall have a two to one safety factor.

Mechanical Joint Restraint:

Restraint Devices for 16 inch AWWA C909 shall consist of multiple gripping wedges incorporated into a follower gland meeting the applicable requirements of ANSI/AWWA C110/A21.10.  The restraint for 16 inch mechanical joint fittings shall be the Series 2016PV.

Restraint Harness:

Restraint Harnesses for 16 inch AWWA C909 shall consist of two restraint rings incorporating multiple gripping wedges per ring, and a sufficient number of bolts shall be used to connect the restraint rings. The combination shall be the Series 2916.

All Series manufactured by EBAA Iron, Inc., or approved equal.

For additional information regarding the 2016PV, including installation instructions refer to the Series 2000PV Product Brochure found at www.ebaa.com.